Transcribed from

Maryland Department of Environment

(Monthly Inspection Reports)

Waste Management Administration

Observation Report

Type of inspection/observation:  	 Landfilling of Municipal Wastes

Site/Facility Name:  		Harford Waste Disposal Center Landfill

Location: 		 Harford

Date:  03/23/07

Remarks:

Inspected today.

- Deep rainwater pooling, mud buildup is seen in an intermediate area next to the homeowner/small truck working face area.  An equipment operator eliminated some of the pooling by grade out during the inspection.

- Leachate breaks are present in the Northeast corner of the landfill bordering the upper portion of the hard road down to the stormwater pond.  Leachate breaks are also seen in upper East slopes facing the stormwater pond.  Leachate in rain pools is noted in the intermediate area in the right side of the hard road to the working faces – showed the foreman the leachate problem, he stated it will be addressed.

- Inadequate intermediate area cover has occurred in the inactive intermediate area on the right side of the hard road to the working faces.  Waste exposure is heavy, needs to be addressed.

- Blown litter on East slopes down to the stormwater pond remains heavy.

- Homeowner bin, brush/yard waste processing stations are maintained.

- No unacceptable waste types.

- All weather access roads are adequate.

Observer:  Ed Class	Person Interviewed:  Gerald P Scanlan