Transcribed from

Maryland Department of Environment

(Monthly Inspection Reports)

Waste Management Administration

Observation Report

Type of inspection/observation:  	 Landfilling of Municipal Wastes

Site/Facility Name:  		Harford Waste Disposal Center Landfill

Location: 		 Harford

Date:  04/26/07

Remarks:

Inspected today.

- A large area of inadequate daily cover is present in an area to the left of the current working face.  Heavy partial to full exposure of solid waste is observed, especially furniture cushions, rugs, boards, general trash.  Soil coverage of exposed wastes was initiated during the inspection.  Note that the same type of heavy solid waste exposure is seen in the back area of the homeowner/small accounts working face.

- Blown litter over East slopes down past the stormwater pond is excessive again, is hung up in brush, trees below the stormwater pond.

- Several leachate breaks are present on upper East area slopes facing the stormwater pond.  Pooled leachate is observed in the N. East corner of the cell near the back hard road in the usual location, is close to going overboard into the water diversion channel.  Leachate must be addressed.

- No unacceptable waste types

- Brush/yard waste processing, homeowner stations are maintained.

Observer:  Ed Class	Person Interviewed:  Gerald P Scanlan