Transcribed from

Maryland Department of Environment

(Monthly Inspection Reports)

Waste Management Administration

Observation Report

Type of inspection/observation:  	 Landfilling of Municipal Wastes

Site/Facility Name:  		Harford Waste Disposal Center Landfill

Location: 		 Harford

Date:  05/05/05

Remarks:

Inspected today.

- Site litter remains heavy, especially on East slopes of the fill, and in brush on South area top slopes facing the brush/yard waste processing facility.

- A Small leachate break has reoccurred on the area above a waste diversion channel described in the inspection report dated 3/7/05.  Leachate has not entered the water diversion channel yet, but is in a position to do so if rain takes place.  This leachate break and another stronger, much larger leachate break located on grassy upper East slopes must be addressed.

- Deep gully erosion is noted on grass covered upper East area slopes.  Erosion must be repaired.

- Brush, yard waste processing, homeowner bin areas are maintained.

- No unacceptable waste types

- Adequate cover types.

Observer:  Ed Class				Person Interviewed:  Gerald Scanlan