Transcribed from

Maryland Department of Environment

(Monthly Inspection Reports)

Waste Management Administration

Observation Report

Type of inspection/observation:  	 Landfilling of Municipal Wastes

Site/Facility Name:  		Harford Waste Disposal Center Landfill

Location: 		 Harford

Date:  05/19/06

Remarks:

Inspected today.

- Some minor slope litter is present but blown litter is under control overall.

- Two small but flowing leachate breaks are present in the East area of the cell.  These are located above the spillway to the outer water diversion channel.  Leachate has pooled near the entrance to the water diversion channel, is close to leaving the cell.  The condition needs to be addressed right away.

- Intermediate area cover remains inadequate in the inactive area to the left of the current working face with heavy mattress popups, board, plastics exposure.

- No unacceptable waste types.

- Brush/yard waste processing, homeowner bin areas are maintained.

Observer:  Ed Class				Person Interviewed:  Gerald Scanlan