Transcribed from

Maryland Department of Environment

(Monthly Inspection Reports)

Waste Management Administration

Observation Report

Type of inspection/observation:  	 Landfilling of Municipal Wastes

Site/Facility Name:  		Harford Waste Disposal Center Landfill

Location: 		 Harford

Date:  07/19/06

Remarks:

Inspected today.

- Daily cover is inadequate in areas adjacent to the current working face where sparsely soil covered wastes are present.  Waste exposure is especially heavy at the East end of the lift.  Soil coverage of wastes began during the inspection.  The permit holder is advised to get them all covered by day’s end of 7/20/06.

- Deep sheet/gully erosion is seen in the intermediate area below where the red fuel oil dumpster is located.  Some underlying waste and liner exposure has occurred.  Erosion with waste exposure is also seen in the inactive intermediate area on the right side of the hard road as it enters the active cells for waste disposal.  Erosion must be addressed.

- Stabilization with seeding is overdue on upper South, East area outer perimeter slopes, bare soil areas above where the current working faces is located.  Gerald Scanlan stated hydro seeding of these is to start soon.

- No unacceptable waste types.

- Rain water with a dark yellow scum in the recycled gasoline tank in the homeowners area is about to spill over the containment wall.  This needs to be pumped down.

- The brush, yard waste processing area is maintained.

Observer:  Ed Class	Person Interviewed:  Gerald Scanlan